- Android BT HAL 库移植

- Driver

- make menuconfig ;选中Enable bluedroid：

- Hal

- 将altobeam包拷贝至hardware目录下。

- hardware\altobeam\libbt\atbm.mk文件将复制fw.bin到指定目录，该目录要与kernel配置一致,默认/vendor/etc/firmwar。fw.bin 位置：hardware\altobeam\libbt\conf\firmware。

- CUR_PATH := hardware/altobeam/libbt - PRODUCT_COPY_FILES += \ - $(CUR_PATH)/conf/firmware/fw.bin:$(TARGET_COPY_OUT_VENDOR)/etc/firmware/fw.bin

- device/rockchip/xxxxx/wifi_bt.mk添加：

diff --git a/device/rockchip/rk3399/wifi_bt.mk b/device/rockchip/rk3399/wifi_bt.mk index 6d53792560..559f0e6811 100644 --- a/device/rockchip/rk3399/wifi_bt.mk +++ b/device/rockchip/rk3399/wifi_bt.mk @@ -39,5 +39,8 @@ # rda587x, #like rtl8188+rda587x # mt6622, #like rtl8188+mt6622 # -BOARD_CONNECTIVITY_VENDOR := Broadcom -BOARD_CONNECTIVITY_MODULE := ap6xxx +#BOARD_CONNECTIVITY_VENDOR := Broadcom +#BOARD_CONNECTIVITY_MODULE := ap6xxx + +BOARD_CONNECTIVITY_VENDOR := AltoBeam +BOARD_CONNECTIVITY_MODULE := atbm606x

- device/rockchip/common/wifi_bt_commod.mk添加：

diff --git a/device/rockchip/common/wifi_bt_common.mk b/device/rockchip/common/wifi_bt_common.mk index 9e157683b6..c6d5d2a2b7 100644 --- a/device/rockchip/common/wifi_bt_common.mk +++ b/device/rockchip/common/wifi_bt_common.mk @@ -37,5 +37,15 @@ BLUETOOTH_ENABLE_FM := false endif endif + +ifeq ($(strip $(BOARD_CONNECTIVITY_VENDOR)), Altobeam) +BOARD_HAVE_BLUETOOTH := true +BOARD_HAVE_BLUETOOTH_ATBM := true + +ifeq ($(strip $(BOARD_CONNECTIVITY_MODULE)), atbm606x) +BLUETOOTH_USE_BPLUS := true +BLUETOOTH_ENABLE_FM := false +endif +endif

- device/rockchip/common/device.mk 添加：

diff --git a/device/rockchip/common/device.mk b/device/rockchip/common/device.mk index 47fe75c4b2..94c3f62cd5 100644 --- a/device/rockchip/common/device.mk +++ b/device/rockchip/common/device.mk @@ -794,6 +794,10 @@ ifeq ($(strip $(BOARD_HAVE_BLUETOOTH_RTK)), true) include hardware/realtek/rtkbt/rtkbt.mk endif +ifeq ($(strip $(BOARD_HAVE_BLUETOOTH_ATBM)), true) +include hardware/altobeam/libbt/atbm.mk +endif + ifeq ($(strip $(TARGET_BOARD_PLATFORM_PRODUCT)), box) include device/rockchip/common/samba/rk31_samba.mk PRODUCT_COPY_FILES += \

- 修改Android/system/core/rootdir/init.rc ，修改/dev/atbm_ioctl权限，添加：

- diff --git a/system/core/rootdir/init.rc b/system/core/rootdir/init.rc - index 0fa9febdd3..20f382beab 100644 - --- a/system/core/rootdir/init.rc - +++ b/system/core/rootdir/init.rc - @@ -820,6 +820,8 @@ on property:security.perf_harden=0 - on property:security.perf_harden=1 - write /proc/sys/kernel/perf_event_paranoid 3 - + chmod 777 /dev/atbm_ioctl - # on shutdown - # In device's init.rc, this trigger can be used to do device-specific actions - # before shutdown. e.g disable watchdog and mask error handling

- system修改,配置可发现与可连接模式。

- diff --git a/system/bt/btif/src/btif_core.cc b/system/bt/btif/src/btif_core.cc - index aa77979acf..036104c2d3 100644 - --- a/system/bt/btif/src/btif_core.cc - +++ b/system/bt/btif/src/btif_core.cc - @@ -929,7 +929,9 @@ bt_status_t btif_set_adapter_property(const bt_property_t* property) { - case BT_SCAN_MODE_CONNECTABLE_DISCOVERABLE: - disc_mode = BTA_DM_GENERAL_DISC; - + disc_mode ¦= BTA_DM_BLE_GENERAL_DISCOVERABLE; - conn_mode = BTA_DM_CONN; - + conn_mode ¦= BTA_DM_BLE_CONNECTABLE; - break; - default:

- Libbt-vendor.so库加载日志，用logcat指令查看，设备打开成功：

- 驱动修改

- 驱动部分只修改了字符设备驱动atbm_ioctl_fops  ，将发到我们协议栈的数据通过文件节点/dev/atbm_ioctl传给bluedroid.

- 流程：

- 在打开文件节点/dev/atbm_ioctl时，配置ble()共存。

- read阻塞。读到数据先放到rx_skb_queue 队列中，再通过bt_read_wait唤醒read读取队        	  列里面的数据。

- 通过atbm_ioctl_write发送hci数据.

- 文件节点的名称修改:

- libbt-vendor.so通过文件描述符读写hci数据的接口：

- Android\hardware\interfaces\bluetooth\1.0\default\h4_protocol.cc

- Android\hardware\interfaces\bluetooth\1.0\default\hci_packetizer.cc

- 自测：

- 测试设备无显示屏情况下，准备工具：

- 调试工具adb.exe

- 安卓投屏工具Scrcpy.exe

- BLE调试工具nRF.Connect.apk

- 手机端安装并打开nRF.Connect.apk 创建一个广播设备

- 设备端：usb连接设备端的otg口

- Win+r 输入cmd 打开命令运行窗口，安装nRF.Connect.apk：

- C:\>adb root & adb remount - adbd is already running as root - remount succeeded - C:\>adb install C:\doc\tool\nRF.Connect.4.24.3.apk - Performing Streamed Install - Success

- 运行Scrcpy.exe投屏，打开nRF.Connect工具，扫描手机设备，连接。

- 连接成功后，获取属性值,测试成功：

- 日志导出

- 查看Android系统日志：logcat

- 查看Kernel日志：dmesg  或 cat /dev/kmsg